FAST NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES

Department of Computer Science

CL3001 - COMPUTER NETWORKS LAB

LAB REPORT 13: DYNAMIC ROUTING PROTOCOLS (RIP, EIGRP & OSPF)

Student ID: 23F-3043

# 1. Introduction & Objectives

Dynamic routing protocols automate route discovery and convergence across large-scale internetworks. Unlike static routing, dynamic protocols adapt to topology changes (link failures, new connections) automatically, exchanging route advertisements between neighboring routers. This lab explores three key protocols:
1. RIP (Routing Information Protocol): A distance-vector protocol using hop count as the metric. Simple but limited to 15 hops.
2. EIGRP (Enhanced Interior Gateway Routing Protocol): Cisco's advanced distance-vector/hybrid protocol using composite metrics (bandwidth, delay, reliability, load). Supports VLSM and rapid convergence.
3. OSPF (Open Shortest Path First): A link-state protocol using Dijkstra's SPF algorithm, supporting VLSM, hierarchical area design, and large-scale enterprise networks with fast convergence.

# 2. TASK 1 — RIP (Routing Information Protocol)

Implement RIP protocol for the network topology provided. The topology consists of three routers interconnected via serial links, with each router serving a local LAN segment containing two PCs. RIP version 2 is used to support classless routing and VLSM.

## Network Topology & IP Addressing Table

| Device | Interface | IP Address | Subnet | Subnet Mask | Gateway / Link |
| --- | --- | --- | --- | --- | --- |
| Router0 (Left) | Se2/0 | 10.0.0.1 | 10.0.0.0/30 | 255.255.255.252 | To Router1 |
| Router0 (Left) | Fa0/0 | 192.168.10.1 | 192.168.10.0/24 | 255.255.255.0 | LAN Gateway |
| Router1 (Middle) | Se2/0 | 10.0.0.2 | 10.0.0.0/30 | 255.255.255.252 | To Router0 |
| Router1 (Middle) | Se3/0 | 11.0.0.1 | 11.0.0.0/30 | 255.255.255.252 | To Router2 |
| Router1 (Middle) | Fa0/0 | 192.168.20.1 | 192.168.20.0/24 | 255.255.255.0 | LAN Gateway |
| Router2 (Right) | Se2/0 | 11.0.0.2 | 11.0.0.0/30 | 255.255.255.252 | To Router1 |
| Router2 (Right) | Fa0/0 | 192.168.30.1 | 192.168.30.0/24 | 255.255.255.0 | LAN Gateway |
| PC0 | Fa0 | 192.168.10.2 | 192.168.10.0/24 | 255.255.255.0 | 192.168.10.1 |
| PC1 | Fa0 | 192.168.10.3 | 192.168.10.0/24 | 255.255.255.0 | 192.168.10.1 |
| PC2 | Fa0 | 192.168.20.2 | 192.168.20.0/24 | 255.255.255.0 | 192.168.20.1 |
| PC3 | Fa0 | 192.168.20.3 | 192.168.20.0/24 | 255.255.255.0 | 192.168.20.1 |
| PC4 | Fa0 | 192.168.30.2 | 192.168.30.0/24 | 255.255.255.0 | 192.168.30.1 |
| PC5 | Fa0 | 192.168.30.3 | 192.168.30.0/24 | 255.255.255.0 | 192.168.30.1 |

Figure 1: Task 1 — RIP Topology in Cisco Packet Tracer

# 3. TASK 2 — EIGRP (Enhanced Interior Gateway Routing Protocol)

Implement EIGRP protocol for the given network topology. EIGRP uses a composite metric based on bandwidth and delay by default (K1=1, K3=1). The topology uses an 8-router ring structure with two internal cross-links, a PC-side LAN and a Server-side LAN. An Autonomous System number of 100 is used for all routers.

## WAN Link Table (Outer Ring)

| Link | Router A | Interface | IP | Router B | Interface | IP | Network |
| --- | --- | --- | --- | --- | --- | --- | --- |
| R0–R1 | R0 | S0/0 | 192.168.0.1 | R1 | S0/0 | 192.168.0.2 | 192.168.0.0/30 |
| R1–R5 | R1 | S0/1 | 192.168.0.5 | R5 | S0/0 | 192.168.0.6 | 192.168.0.4/30 |
| R5–R7 | R5 | S0/1 | 192.168.0.9 | R7 | S0/0 | 192.168.0.10 | 192.168.0.8/30 |
| R7–R6 | R7 | S0/1 | 192.168.0.13 | R6 | S0/0 | 192.168.0.14 | 192.168.0.12/30 |
| R6–R3 | R6 | S0/1 | 192.168.0.17 | R3 | S0/0 | 192.168.0.18 | 192.168.0.16/30 |
| R3–R0 | R3 | S0/1 | 192.168.0.21 | R0 | S0/1 | 192.168.0.22 | 192.168.0.20/30 |

## Middle Path Links (Cross Links)

| Link | Router A | IP | Router B | IP | Network |
| --- | --- | --- | --- | --- | --- |
| R3–R4 | R3 | 192.168.1.1 | R4 | 192.168.1.2 | 192.168.1.0/30 |
| R4–R5 | R4 | 192.168.1.5 | R5 | 192.168.1.6 | 192.168.1.4/30 |

### PC-Side LAN

| Device | IP | Mask | Gateway |
| --- | --- | --- | --- |
| Router3 Fa0/0 | 10.0.0.1 | 255.255.255.0 | — |
| PC | 10.0.0.2 | 255.255.255.0 | 10.0.0.1 |

### Server-Side LAN

| Device | IP | Mask | Gateway |
| --- | --- | --- | --- |
| Router5 Fa0/0 | 20.0.0.1 | 255.255.255.0 | — |
| Server | 20.0.0.2 | 255.255.255.0 | 20.0.0.1 |

### EIGRP Router Configuration CLI Commands

enable conf t router eigrp 100 no auto-summary network 192.168.0.0 network 192.168.1.0 network 192.168.2.0 network 10.0.0.0 network 20.0.0.0

Figure 2: Task 2 — EIGRP Topology Overview in Cisco Packet Tracer

Figure 3: Task 2 — EIGRP Successful Ping Verification Between PC and Server

# 4. TASK 3 — OSPF with VLSM Subnetting

Configure a Packet Tracer network topology utilizing Class A IP address 100.0.0.0 and default mask 255.0.0.0. Subnets are designed using VLSM to accommodate 4 LANs with varying host counts (4000, 3000, 2500, and 1800 hosts), plus 4 WAN serial links (/30 subnets). OSPF Area 0 is used for all routers to ensure a flat, single-area dynamic routing domain.

## VLSM LAN Subnet Allocation Table

| Purpose | Network | Mask |
| --- | --- | --- |
| LAN (4000 hosts) | 100.0.0.0/20 | 255.255.240.0 |
| LAN (3000 hosts) | 100.0.16.0/20 | 255.255.240.0 |
| LAN (2500 hosts) | 100.0.32.0/20 | 255.255.240.0 |
| LAN (1800 hosts) | 100.0.48.0/21 | 255.255.248.0 |

## WAN Serial Link Subnets (/30)

| Link | Network | IPs |
| --- | --- | --- |
| R0–R1 | 100.0.56.0/30 | .1 / .2 |
| R0–R2 | 100.0.56.4/30 | .5 / .6 |
| R1–R3 | 100.0.56.8/30 | .9 / .10 |
| R2–R3 | 100.0.56.12/30 | .13 / .14 |

### OSPF Router Configuration CLI Commands

router ospf 1 network 100.0.0.0 0.255.255.255 area 0

Figure 4: Task 3 — OSPF Topology with VLSM Subnets in Cisco Packet Tracer

Figure 5: Task 3 — OSPF Verification Ping Results