CL3001

Computer Networks - Lab
 Lab 12 & Week-12

IP Addressing

Objectives:

- To design and implement IP addressing schemes, subnetting, and VLSM in Cisco Packet Tracer.

For the given IPs fill the following table for first 2 subnets.

- IP = 100.0.0.0   for  3000 subnets

| Subnet no. | Subnet ID | Subnet Mask | 1st usable IP address | 2nd usable IP address | Last usable IP address | Broadcast address |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100.0.0.0 | 255.255.240.0 | 100.0.0.1 | 100.0.0.2 | 100.0.15.254 | 100.0.15.255 |
| 2 | 100.0.16.0 | 255.255.240.0 | 100.0.16.1 | 100.0.16.2 | 100.0.31.254 | 100.0.31.255 |

- IP = 150.89.0.0  for 1800 subnets

| Subnet no. | Subnet ID | Subnet Mask | 1st usable | 2nd usable | Last usable | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 150.89.0.0 | 255.255.255.224 | 150.89.0.1 | 150.89.0.2 | 150.89.0.30 | 150.89.0.31 |
| 2 | 150.89.0.32 | 255.255.255.224 | 150.89.0.33 | 150.89.0.34 | 150.89.0.62 | 150.89.0.63 |

- IP = 203.45.67.0  for 10 subnets

| Subnet no. | Subnet ID | Subnet Mask | 1st usable | 2nd usable | Last usable | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 203.45.67.0 | 255.255.255.240 | 203.45.67.1 | 203.45.67.2 | 203.45.67.14 | 203.45.67.15 |
| 2 | 203.45.67.16 | 255.255.255.240 | 203.45.67.17 | 203.45.67.18 | 203.45.67.30 | 203.45.67.31 |

Subnetting

For the following IPs do the subnetting as per the no. of hosts given and fill the following table.

- IP = 87.0.0.0  for 20000, 124000 , 90000 , 80000 hosts

| Subnet no. | Subnet ID | Subnet Mask | 1st usable IP address | 2nd usable IP address | Last usable IP address | Broadcast address | No. of hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 124000 | 87.0.0.0 | 255.254.0.0 | 87.0.0.1 | 87.0.0.2 | 87.1.255.254 | 87.1.255.255 | 131070 |
| 90000 | 87.2.0.0 | 255.254.0.0 | 87.2.0.1 | 87.2.0.2 | 87.3.255.254 | 87.3.255.255 | 131070 |
| 80000 | 87.4.0.0 | 255.254.0.0 | 87.4.0.1 | 87.4.0.2 | 87.5.255.254 | 87.5.255.255 | 131070 |
| 20000 | 87.6.0.0 | 255.255.128.0 | 87.6.0.1 | 87.6.0.2 | 87.6.127.254 | 87.6.127.255 | 32766 |

- IP = 156.78.0.0 for   20000 , 8000 , 16000 , 500 hosts

| Subnet no. | Subnet ID | Subnet Mask | 1st usable IP address | 2nd usable IP address | Last usable IP address | Broadcast address | No. of hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16000 | 156.78.0.0 | 255.255.128.0 (/17) | 156.78.0.1 | 156.78.0.2 | 156.78.127.254 | 156.78.127.255 | 20000 |
| 20000 | 156.78.128.0 | 255.255.192.0 (/18) | 156.78.128.1 | 156.78.128.2 | 156.78.191.254 | 156.78.191.255 | 16000 |
| 8000 | 156.78.192.0 | 255.255.224.0 (/19) | 156.78.192.1 | 156.78.192.2 | 156.78.223.254 | 156.78.223.255 | 8000 |
| 500 | 156.78.224.0 | 255.255.254.0 (/23) | 156.78.224.1 | 156.78.224.2 | 156.78.225.254 | 156.78.225.255 | 500 |

- IP = 192.168.1.0 for 50 , 12, 30 , 19 hosts

| Subnet no. | Subnet ID | Subnet Mask | 1st usable IP address | 2nd usable IP address | Last usable IP address | Broadcast address | No. of hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 192.168.1.0 | 255.255.255.192 (/26) | 192.168.1.1 | 192.168.1.2 | 192.168.1.62 | 192.168.1.63 | 50 |
| 30 | 192.168.1.64 | 255.255.255.224 (/27) | 192.168.1.65 | 192.168.1.66 | 192.168.1.94 | 192.168.1.95 | 30 |
| 19 | 192.168.1.96 | 255.255.255.224 (/27) | 192.168.1.97 | 192.168.1.98 | 192.168.1.126 | 192.168.1.127 | 19 |
| 12 | 192.168.1.128 | 255.255.255.240 (/28) | 192.168.1.129 | 192.168.1.130 | 192.168.1.142 | 192.168.1.143 | 12 |

Task 1:

You have learned IP Addressing classes, CIDR and their fix length subnet mask (FLSM) and variable length subnet mask VLSM. Also, you have practice of static routing between networks in previous labs. Now you are required to configure the given topology using the above-mentioned concepts. For this topology is given.  IP: 196.10.0.0 & subnet mask: 255.255.255.0. You are required to create 5 subnetworks using FLSM technique of the given IP address of class C.

Clearly perform each step from scratch to generate the required subnets and their IP address, now configure topology using these Ips and subnets and perform topology static routing testing. Each step has marks, so be careful.

# Subnet Table:

| Subnet | Network ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 196.10.0.0 | 255.255.255.224 | 196.10.0.1 | 196.10.0.2 | 196.10.0.30 | 196.10.0.31 |
| 2 | 196.10.0.32 | 255.255.255.224 | 196.10.0.33 | 196.10.0.34 | 196.10.0.62 | 196.10.0.63 |
| 3 | 196.10.0.64 | 255.255.255.224 | 196.10.0.65 | 196.10.0.66 | 196.10.0.94 | 196.10.0.95 |
| 4 | 196.10.0.96 | 255.255.255.224 | 196.10.0.97 | 196.10.0.98 | 196.10.0.126 | 196.10.0.127 |
| 5 | 196.10.0.128 | 255.255.255.224 | 196.10.0.129 | 196.10.0.130 | 196.10.0.158 | 196.10.0.159 |

Topology:

Ip configuration at router 1(named as router 4 in packet tracer):

Ip configuration at router 2(named as router 	5 in packet tracer):

PC0 configuration:

PC2 configuration:

Pinging from PC0 to PC2

Task 2:

You have learned IP Addressing classes, CIDR and their fix length subnet mask (FLSM) and variable length subnet mask VLSM. Also, you have practice of static routing between networks in previous labs. Now you are required to configure the given topology using the above-mentioned concepts. For this topology is given.  IP: 10.0.0.0 & subnet mask: 255.0.0.0. You are required to create subnetworks but with different number of required Host IDs using VLSM technique of the given IP address of class A.

Clearly perform each step from scratch to generate the required subnets and their IP address, now configure topology using these Ips and subnets and perform topology static routing testing. Each step has marks, so be careful.

# VLSM RULE:

| LAN | Hosts Needed |
| --- | --- |
| LAN 1 | 2000 |
| LAN 2 | 1000 |
| LAN 3 | 400 |
| WAN (Serial) | 2 |

# FINAL SUBNET TABLE:

# LAN (2000 Hosts):

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.0.0 |
| Subnet Mask | 255.255.248.0 (/21) |
| First Usable IP | 10.0.0.1 |
| Last Usable IP | 10.0.7.254 |
| Broadcast | 10.0.7.255 |
| Total Hosts | 2046 |

# LAN (1000 Hosts):

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.8.0 |
| Subnet Mask | 255.255.252.0 (/22) |
| First Usable IP | 10.0.8.1 |
| Last Usable IP | 10.0.11.254 |
| Broadcast | 10.0.11.255 |
| Total Hosts | 1022 |

# LAN (400 Hosts):

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.12.0 |
| Subnet Mask | 255.255.254.0 (/23) |
| First Usable IP | 10.0.12.1 |
| Last Usable IP | 10.0.13.254 |
| Broadcast | 10.0.13.255 |
| Total Hosts | 510 |

# WAN / SERIAL LINK:

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.14.0 |
| Subnet Mask | 255.255.255.252 (/30) |
| First Usable IP | 10.0.14.1 |
| Last Usable IP | 10.0.14.2 |
| Broadcast | 10.0.14.3 |
| Total Hosts | 2 |

# SUMMARY TABLE:

| Network | Purpose | Subnet |
| --- | --- | --- |
| 10.0.0.0/21 | Right LAN (2000 hosts) | Router → Switch |
| 10.0.8.0/22 | Left LAN (1000 hosts) | Router → Switch |
| 10.0.12.0/23 | Middle LAN (400 hosts) | Switch shared |
| 10.0.14.0/30 | WAN link | Router ↔ Router |

# Topology:

Ip configuration router 1(named as router 4 in packet tracer):

Ip configuration router 2(named as router 5 in packet tracer):

# PC0 configuration:

PC4 configuration:

Pinging From PC0 to PC4:

# Pinging From PC4 to PC0: