CL3001

Computer Networks - Lab
 Lab 11 & Week-11

Objectives:

- To configure and analyze advanced socket functions: getsockopt(), setsockopt(), select(), and poll()

Background

In the real world, servers do not just "send and receive" — they tune their own behavior to stay reliable, fast, and scalable. Four advanced socket functions make this possible:

- getsockopt() — Reads the current value of a socket option (e.g., buffer size, timeout). Like checking the settings menu of a socket.

- setsockopt() — Writes (changes) a socket option. Like changing settings to make the socket behave differently.

- select() — Watches multiple sockets at once and tells you which one is ready. Imagine a receptionist watching 10 phones — they pick up whichever rings first.

- poll() — Does the same job as select() but scales better. Think of it as an upgraded receptionist who can watch hundreds of phones without slowing down.

In this lab, you will observe all four functions in action using Cisco Packet Tracer’s built-in HTTP, DNS, and FTP services — which use these mechanisms internally.

How to Build the Network — Physical Setup

Build this topology in Cisco Packet Tracer before answering any questions.

Step 1:  Physical Connections

- Open Cisco Packet Tracer. Add 1 Server, 1 Switch, and 3 PCs to the workspace.

- Connect every device to the Switch using Copper Straight-Through cables.

Step 2:  IP Addressing

Assign the following static IP addresses (Desktop → IP Configuration on each PC):

| Device | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- |
| Server0 | 192.168.1.10 | 255.255.255.0 | 192.168.1.10 |
| PC-A | 192.168.1.1 | 255.255.255.0 | 192.168.1.10 |
| PC-B | 192.168.1.2 | 255.255.255.0 | 192.168.1.10 |
| PC-C | 192.168.1.3 | 255.255.255.0 | 192.168.1.10 |

Step 3:  Enable Server Services (this simulates setsockopt in action)

- Click Server0 → Services tab.

- HTTP: Click HTTP → Turn HTTP On → Turn HTTPS On. This opens socket on Port 80 (TCP). The server internally uses SO_REUSEADDR so it can restart on the same port.

- DNS: Click DNS → Turn On → Add record: Name = lab.com, Address = 192.168.1.10. DNS uses Port 53 (UDP).

- FTP: Click FTP → Turn On. FTP uses Port 21 (TCP). We will use this to simulate a third concurrent socket — like poll() monitoring a third file descriptor.

Step 4:  Verify Connectivity

- On PC-A: Desktop → Command Prompt → type:   ping 192.168.1.10

- On PC-B: Desktop → Command Prompt → type:   ping 192.168.1.10

- You should see "Reply from 192.168.1.10". If not, recheck cables and IP addresses.

Step 5:  Configure DNS on all PCs

- On each PC: Desktop → IP Configuration → set DNS Server = 192.168.1.10

Part A: getsockopt() and setsockopt() — Reading and Writing Socket Options using TCP and UDP

Concept:  Every socket in an OS has hidden settings — like buffer sizes, timeout values, and address-reuse flags. setsockopt() writes these. getsockopt() reads them back. Cisco Packet Tracer’s HTTP server uses SO_REUSEADDR internally (so it can restart on Port 80 immediately) and SO_KEEPALIVE (so idle HTTP connections stay alive). You will observe the effects of both.

How to Observe:

- Switch to Simulation Mode (bottom-right clock icon).

- Click Edit Filters → check only: TCP, HTTP.

- On PC-A: Desktop → Web Browser → type 192.168.1.10 → press Go.

- Click Capture/Forward slowly. Watch the PDU envelopes travel.

- Click any PDU → open the PDU window → go to the Outbound PDU Details tab → scroll to TCP Header.

Key things to look for in Layer 4 (TCP Header): Window Size (receive buffer), Flags, Sequence Number, ACK Number.

Programming Tab — Code A: setsockopt() and getsockopt() Demonstration Using TCP

Server Code — setsockopt() and getsockopt()  |  Run on: Server0

from tcp import *

from time import *

# ── setsockopt() SIMULATION ─────────────────────────────────────

# PT does not expose setOption/getOption on TCPServer.

# We simulate setsockopt by storing options as plain variables.

# In a real OS: setsockopt(sock, SOL_SOCKET, SO_REUSEADDR, &1, sizeof(1))

port = 12000

server = TCPServer()

# Simulated setsockopt() calls

socket_options = {

'SO_REUSEADDR': 1,   # allow port reuse after server restart

'SO_RCVTIMEO':  8,   # receive timeout = 8 seconds

}

# ── getsockopt() SIMULATION ─────────────────────────────────────

# In real C: getsockopt(sock, SOL_SOCKET, SO_REUSEADDR, &val, &len)

reuse = socket_options['SO_REUSEADDR']

timeo = socket_options['SO_RCVTIMEO']

print('=== getsockopt() READ-BACK ===')

print('SO_REUSEADDR = ' + str(reuse) + '  (1=ON, 0=OFF)')

print('SO_RCVTIMEO  = ' + str(timeo) + ' seconds')

print('===============================')

# ── Server logic ─────────────────────────────────────────────────

client_ref = [None]

def onNewClient(c):

client_ref[0] = c

ip = c.remoteIP()

print('')

print('[setsockopt] New client from: ' + ip)

print('[getsockopt] SO_REUSEADDR=' + str(reuse) + ' SO_RCVTIMEO=' + str(timeo) + 's')

c.onReceive(onReceive)

msg_count = [0]

def onReceive(data):

msg_count[0] += 1

print('[Recv #' + str(msg_count[0]) + '] ' + data)

client_ref[0].send('Echo from server: ' + data)

server.onNewClient(onNewClient)

server.listen(port)

print('Server listening on port ' + str(port) + '...')

print('SO_REUSEADDR lets this server restart on the same port immediately.')

while True:

sleep(1)

Client Code — setsockopt() / getsockopt()  |  Run on: PC-A

from tcp import *

from time import *

serverIP = '192.168.1.10'   # Server0 IP — change if needed

port     = 12000

client   = TCPClient()

# ── setsockopt() SIMULATION ──────────────────────────────────────

# PT does not expose setOption/getOption on TCPClient.

# We simulate setsockopt by storing the option in a plain dictionary.

# In real C: setsockopt(sock, SOL_SOCKET, SO_RCVTIMEO, &tv, sizeof(tv))

socket_options = {

'SO_RCVTIMEO': 5,   # receive timeout = 5 seconds

'SO_REUSEADDR': 1,   # allow port reuse

}

# ── getsockopt() SIMULATION ──────────────────────────────────────

# In real C: getsockopt(sock, SOL_SOCKET, SO_RCVTIMEO, &val, &len)

val = socket_options['SO_RCVTIMEO']

print('=== Client getsockopt() READ-BACK ===')

print('SO_RCVTIMEO  = ' + str(val) + ' seconds  (receive timeout)')

print('SO_REUSEADDR = ' + str(socket_options['SO_REUSEADDR']) + '  (1=ON, 0=OFF)')

print('======================================')

# ── Callback handlers ─────────────────────────────────────────────

def onReceive(data):

print('[Reply] ' + data)

ready = [False]

def onConnect(type):

if type == 0:

ready[0] = True

print('Connected to server at ' + serverIP + ':' + str(port))

print('[setsockopt effect] SO_RCVTIMEO active: will timeout after ' + str(val) + 's if no reply')

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

# ── Wait for connection (respects simulated SO_RCVTIMEO) ──────────

count = 0

while not ready[0] and count < val:   # use our simulated timeout value

sleep(1)

count += 1

if ready[0]:

sleep(1)

print('Sending message 1...')

client.send('Hello - testing setsockopt SO_REUSEADDR')

sleep(3)

print('Sending message 2...')

client.send('Second message - check window size in PDU')

sleep(2)

print('Done. Check server output for getsockopt readback.')

else:

print('Connection failed - SO_RCVTIMEO expired after ' + str(val) + 's')

Questions — getsockopt() and setsockopt()

A1.  Open the TCP header of the FIRST packet sent from PC-A to the Server. What is the value in the Window Size field? This value is controlled by SO_RCVBUF (receive buffer size). Write it below.

A2.  The HTTP server on Cisco Packet Tracer uses SO_REUSEADDR. What problem does this option solve? If SO_REUSEADDR were OFF and the server restarted, what error would appear when it tried to listen on Port 80 again?

A3.  Look at the TCP header of the response packet from the Server to PC-A. Is there a Keep-Alive flag or option visible? What socket option (SO_KEEPALIVE) does this correspond to, and why does HTTP need it?

A4.  In the PDU window, click Layer 4. Compare the Window Size in the first packet vs. the third or fourth packet. Did the window size change? What does a changing window size tell you about the socket’s internal buffer management?

A5.  If a programmer calls setsockopt(sock, SOL_SOCKET, SO_RCVBUF, &val, sizeof(val)) and then immediately calls getsockopt() to read it back, the OS may return DOUBLE the value they set. Why does this happen? (Hint: the OS adds its own internal overhead buffer.)

Part B: select() — Watching Multiple Sockets Simultaneously Using TCP and UDP both

Concept:  select() lets one server process watch many sockets without threading. It takes three sets of file descriptors: read-set (sockets with incoming data), write-set (sockets ready to send), and error-set (sockets with errors). It blocks until any one of them becomes ready. In this experiment, you will simulate select() by having the server handle two services simultaneously — HTTP (Port 80) and DNS (Port 53) — and observe which one responds first.

How to Observe:

- Keep Simulation Mode ON. Click Edit Filters → check: TCP, UDP, HTTP, DNS.

- Open Web Browser on PC-A → type lab.com → press Go. (This sends a DNS query first, then HTTP.)

- While that is pending — open PC-B → Desktop → Command Prompt → type:  nslookup lab.com

- Press Play (or Capture/Forward step by step). Watch both packets travel simultaneously.

- Click each PDU and compare the Layer 4 headers.

Questions — select()

B1.  When PC-A types lab.com in the browser, two requests are generated in order. What is the FIRST request sent (DNS or HTTP)? What port does it go to? What protocol (TCP or UDP) does it use?

B2.  In Simulation Mode, you can see both a DNS packet (from PC-A or PC-B) and an HTTP packet at the same time. In a real server using select(), both of these would appear in the read-set. Describe in your own words: what does select() do when BOTH sockets become ready at the same time?

B3.  Click the DNS PDU (yellow envelope with UDP). Open PDU Details → Layer 4. Now click the HTTP PDU → Layer 4. Fill in the table:

| Field | DNS Packet (UDP) | HTTP Packet (TCP) |
| --- | --- | --- |
| Protocol |  |  |
| Source Port |  |  |
| Destination Port |  |  |
| Sequence Number |  |  |
| ACK Field Present? |  |  |

B4.  select() has a well-known limitation: it uses a fixed-size array called fd_set with a maximum size of FD_SETSIZE (usually 1024). What does this mean for a busy web server trying to handle 5000 simultaneous clients using select()?

B5.  In Simulation Mode, which packet arrived at the server FIRST — the DNS query from PC-B or the HTTP request from PC-A? Does the arrival order affect the correctness of the server’s response? Explain.

B6.  The Server handles both HTTP and DNS without crashing or mixing up responses. What information does the server use to identify WHICH client sent WHICH request and where to send the reply? (Think: IP address + port number.)

Programming Tab — Code B: select() Demonstration

Server Code — select() Simulation  |  Run on: Server0

from tcp import *

from time import *

# ── select() SIMULATION ──────────────────────────────────────────

# In a real OS, select() watches multiple file descriptors (fds).

# Each onReceive() callback here fires when a socket enters read_fds.

# select_log tracks which fd became ready and when.

port       = 12000

server     = TCPServer()

select_log = []          # tracks which fd became ready

clients    = {}          # fd_label -> client object

conn_count = [0]

def make_select_handler(fd_label):

"""

Returns an onReceive callback that logs the select() event.

In a real select() loop, this fires when fd is in read_fds.

"""

def on_ready(data):

event_num = len(select_log) + 1

select_log.append(fd_label)

print('')

print('=== select() returned: fd=' + fd_label + ' is in read_fds ===')

print('    Event #' + str(event_num) + '  |  Data: ' + data)

print('    -> Dispatching handler for ' + fd_label)

clients[fd_label].send('[select-reply to ' + fd_label + ']: ' + data)

return on_ready

def onNewClient(c):

conn_count[0] += 1

label = 'fd' + str(conn_count[0]) + '_' + c.remoteIP()

clients[label] = c

c.onReceive(make_select_handler(label))

print('[select] New fd registered: ' + label)

print('[select] Total fds in read_fds set: ' + str(conn_count[0]))

server.onNewClient(onNewClient)

server.listen(port)

print('select()-style Server ready on port ' + str(port))

print('Monitoring read_fds | write_fds | error_fds...')

print('Waiting for clients to appear in read_fds...')

print('')

# Polling loop simulates select() blocking with a 1s timeout

tick = [0]

while True:

sleep(1)

tick[0] += 1

if tick[0] % 5 == 0:

print('[select timeout] tick=' + str(tick[0]) +

' | total events so far: ' + str(len(select_log)))

Client Code — select()  |  Run on: PC-A

from tcp import *

from time import *

serverIP = '192.168.1.10'

port     = 12000

client   = TCPClient()

ready    = [False]

def onReceive(data):

print('[PC-A select-reply]: ' + data)

def onConnect(t):

if t == 0: ready[0] = True

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

count = 0

while not ready[0] and count < 15:

sleep(1); count += 1

if ready[0]:

sleep(1)

# This message enters the server's read_fds set -> select() fires

client.send('PC-A: Message-1 entering select read_fds')

sleep(4)   # delay so PC-B can also connect and both appear in read_fds

client.send('PC-A: Message-2 entering select read_fds')

sleep(2)

print('PC-A done.')

Client Code — select()  |  Run on: PC-B

from tcp import *

from time import *

serverIP = '192.168.1.2'

port     = 12000

client   = TCPClient()

ready    = [False]

def onReceive(data):

print('[PC-B select-reply]: ' + data)

def onConnect(t):

if t == 0: ready[0] = True

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

count = 0

while not ready[0] and count < 15:

sleep(1); count += 1

if ready[0]:

sleep(3)   # intentional delay so both clients overlap in read_fds

client.send('PC-B: Message-1 entering select read_fds')

sleep(2)

client.send('PC-B: Message-2 entering select read_fds')

sleep(2)

print('PC-B done.')

Part C: poll() — Scalable Multi-Socket Monitoring using TCP and UDP

Concept:  poll() solves the FD_SETSIZE limitation of select(). Instead of three bit-sets, poll() uses an array of pollfd structures. Each entry stores: the socket descriptor (fd), the events you want to monitor (POLLIN for data, POLLOUT for write-ready), and the returned events (revents) after poll() returns. In this experiment, you will trigger THREE simultaneous services — HTTP, DNS, and FTP — and observe how the server handles all three, just like poll() monitoring three file descriptors in its pollfd array.

How to Observe:

- In Simulation Mode → Edit Filters → check: TCP, UDP, HTTP, DNS, FTP.

- On PC-A: Desktop → Web Browser → type lab.com → Go.

- On PC-B: Desktop → Command Prompt → nslookup lab.com

- On PC-C: Desktop → Command Prompt → ftp 192.168.1.10

- Click Capture/Forward. All three packet types should appear simultaneously.

- Click each PDU one by one and read the Layer 4 details.

Server Code — poll() Simulation  |  Run on: Server0

from tcp import *

from time import *

# ── poll() CONCEPT ──────────────────────────────────────────────

# Real poll() signature:

#   poll(pollfd_array, nfds, timeout_ms)

#

# pollfd structure per socket:

#   fd      -> the socket file descriptor

#   events  -> what to watch: POLLIN | POLLOUT | POLLERR

#   revents -> what actually happened (filled by kernel after poll())

#

# poll() advantage over select():

#   - No FD_SETSIZE limit (can watch thousands of fds)

#   - revents tells you EXACTLY what happened on each fd

#   - More readable: one array instead of three bit-sets

port = 12000

server = TCPServer()

# pollfd_table simulates the poll() array

# Each entry: { 'fd': ip, 'events': 'POLLIN', 'revents': '...', 'count': N }

pollfd_table = {}

clients      = {}

conn_count   = [0]

def make_poll_handler(fd_key):

"""

Simulates poll() firing when revents = POLLIN for this fd.

In real code this executes after poll() returns POLLIN in revents.

"""

def on_pollin(data):

if fd_key not in pollfd_table:

return

pollfd_table[fd_key]['count']   += 1

pollfd_table[fd_key]['revents']  = 'POLLIN'

n = str(pollfd_table[fd_key]['count'])

print('')

print('[ poll() ] fd=' + fd_key + ' | revents=POLLIN | msg#' + n)

print('           data: ' + data)

clients[fd_key].send('[poll-reply fd=' + fd_key + ']: ' + data)

return on_pollin

def onNewClient(c):

conn_count[0] += 1

ip  = c.remoteIP()

key = 'fd' + str(conn_count[0]) + ':' + ip

clients[key]     = c

pollfd_table[key] = {

'fd':      key,

'events':  'POLLIN',    # we want to monitor incoming data

'revents': 'POLLNONE',  # nothing yet

'count':   0

}

c.onReceive(make_poll_handler(key))

print('[poll] REGISTERED fd=' + key)

print('[poll] events=POLLIN | revents=POLLNONE (waiting...)')

print('[poll] Total fds in pollfd array: ' + str(conn_count[0]))

server.onNewClient(onNewClient)

server.listen(port)

print('poll()-style Server running on port ' + str(port))

print('pollfd array will grow as clients connect.')

print('')

# Print pollfd table snapshot every 6 seconds

tick = [0]

while True:

sleep(1)

tick[0] += 1

if tick[0] % 6 == 0:

print('')

print('===== poll() pollfd TABLE SNAPSHOT (tick=' + str(tick[0]) + ') =====')

for k in pollfd_table:

e  = pollfd_table[k]

print('  fd=' + e['fd'] +

' | events=' + e['events'] +

' | revents=' + e['revents'] +

' | msg_count=' + str(e['count']))

print('======================================================')

Client Code — poll()  |  Run on: PC-A

from tcp import *

from time import *

serverIP = '192.168.1.2'

port     = 12000

client   = TCPClient()

ready    = [False]

def onReceive(data): print('[PC-A] ' + data)

def onConnect(t):

if t == 0: ready[0] = True

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

count = 0

while not ready[0] and count < 15:

sleep(1); count += 1

if ready[0]:

sleep(1)

# Each send() causes revents=POLLIN on the server's pollfd entry

client.send('PC-A POLLIN trigger 1')

sleep(4)

client.send('PC-A POLLIN trigger 2')

sleep(2)

print('PC-A done.')

Client Code — poll()  |  Run on: PC-B

from tcp import *

from time import *

serverIP = '192.168.1.2'

port     = 12000

client   = TCPClient()

ready    = [False]

def onReceive(data): print('[PC-B] ' + data)

def onConnect(t):

if t == 0: ready[0] = True

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

count = 0

while not ready[0] and count < 15:

sleep(1); count += 1

if ready[0]:

sleep(3)

client.send('PC-B POLLIN trigger 1')

sleep(2)

client.send('PC-B POLLIN trigger 2')

sleep(2)

print('PC-B done.')

Client Code — poll()  |  Run on: PC-C

from tcp import *

from time import *

serverIP = '192.168.1.2'

port     = 12000

client   = TCPClient()

ready    = [False]

def onReceive(data): print('[PC-C] ' + data)

def onConnect(t):

if t == 0: ready[0] = True

client.onReceive(onReceive)

client.onConnectionChange(onConnect)

client.connect(serverIP, port)

count = 0

while not ready[0] and count < 15:

sleep(1); count += 1

if ready[0]:

sleep(5)   # delay to make all three overlap in pollfd table

client.send('PC-C POLLIN trigger 1')

sleep(2)

client.send('PC-C POLLIN trigger 2')

sleep(2)

print('PC-C done.')

Questions — poll()

C1.  In this experiment, three services ran at once: HTTP (TCP/80), DNS (UDP/53), and FTP (TCP/21). In a real poll() call, each of these would be a separate entry in the pollfd array. Fill in the table below based on your PDU observations:

| Service | Protocol | Port | poll() Event | PC that sent it |
| --- | --- | --- | --- | --- |
| HTTP |  |  | POLLIN |  |
| DNS |  |  | POLLIN |  |
| FTP |  |  | POLLIN |  |

C2.  Each pollfd structure has a field called revents which the kernel fills in AFTER poll() returns. In your observation, which service’s packet arrived at the server first? If this were a real poll() call, which pollfd entry would have POLLIN set in its revents field first?

C3.  FTP uses TWO connections: a control connection (Port 21) and a data connection (Port 20). How does this affect the number of entries needed in a poll() array for one FTP client? Did you observe two separate PDUs for FTP in Simulation Mode?

C4.  poll() does not have the FD_SETSIZE limitation of select(). Based on what you observed in this experiment (3 services, 3 clients), explain in your own words WHY poll() would be a better choice than select() if this server needed to handle 3000 simultaneous clients instead of 3.

C5.  In Simulation Mode, pause at the moment all three PDUs (HTTP, DNS, FTP) are visible on the canvas at the same time. Describe exactly what you see. How many PDU envelopes are visible? What colors are they? Where are they located on the network?