FAST NATIONAL UNIVERSITY OF COMPUTER AND EMERGING SCIENCES

Department of Computer Science

CL3001 - COMPUTER NETWORKS LAB

LAB REPORT 12: IP SUBNETTING & STATIC ROUTING (FLSM & VLSM)

Student ID: 23F-3043

# 1. Introduction & Objectives

IP subnetting is a foundational concept in computer network design that partition a single physical network into multiple logical sub-networks (subnets). Subnetting controls broadcast traffic, enhances security boundaries, and optimizes address space efficiency. This lab exercises two primary subnetting techniques:
1. Fixed Length Subnet Masking (FLSM): Allocates identical network address block size and subnet mask for each sub-network regardless of actual host counts.
2. Variable Length Subnet Masking (VLSM): Allocates subnets of varying sizes and masks based directly on individual sub-network host requirements, significantly reducing IP address wastage.

# 2. FLSM & VLSM Address Calculations

## FLSM Exercise 1: IP = 100.0.0.0 for 3000 subnets

To create 3000 subnets from Class A IP 100.0.0.0:
• Number of subnet bits needed: 2^12 = 4096 >= 3000 (Borrow 12 bits from host portion).
• New Subnet Mask: /20 (255.255.240.0)
• Subnets increment: 16 in the third octet.

| Subnet No. | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 100.0.0.0 | 255.255.240.0 | 100.0.0.1 | 100.0.0.2 | 100.0.15.254 | 100.0.15.255 |
| 2 | 100.0.16.0 | 255.255.240.0 | 100.0.16.1 | 100.0.16.2 | 100.0.31.254 | 100.0.31.255 |

## FLSM Exercise 2: IP = 150.89.0.0 for 1800 subnets

To create 1800 subnets from Class B IP 150.89.0.0:
• Number of subnet bits needed: 2^11 = 2048 >= 1800 (Borrow 11 bits from host portion).
• New Subnet Mask: /27 (255.255.255.224)
• Subnets increment: 32 in the fourth octet.

| Subnet No. | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 150.89.0.0 | 255.255.255.224 | 150.89.0.1 | 150.89.0.2 | 150.89.0.30 | 150.89.0.31 |
| 2 | 150.89.0.32 | 255.255.255.224 | 150.89.0.33 | 150.89.0.34 | 150.89.0.62 | 150.89.0.63 |

## FLSM Exercise 3: IP = 203.45.67.0 for 10 subnets

To create 10 subnets from Class C IP 203.45.67.0:
• Number of subnet bits needed: 2^4 = 16 >= 10 (Borrow 4 bits from host portion).
• New Subnet Mask: /28 (255.255.255.240)
• Subnets increment: 16 in the fourth octet.

| Subnet No. | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 203.45.67.0 | 255.255.255.240 | 203.45.67.1 | 203.45.67.2 | 203.45.67.14 | 203.45.67.15 |
| 2 | 203.45.67.16 | 255.255.255.240 | 203.45.67.17 | 203.45.67.18 | 203.45.67.30 | 203.45.67.31 |

## VLSM Exercise 1: IP = 87.0.0.0 for 20000, 124000, 90000, 80000 hosts

For VLSM calculations, networks are sorted in descending order of host requirements to maximize address density:
1. 124,000 hosts: Needs 2^17 = 131,072 IPs -> /17 mask (255.254.0.0)
2. 90,000 hosts: Needs 2^17 = 131,072 IPs -> /17 mask (255.254.0.0)
3. 80,000 hosts: Needs 2^17 = 131,072 IPs -> /17 mask (255.254.0.0)
4. 20,000 hosts: Needs 2^15 = 32,768 IPs -> /15 mask (255.255.128.0)

| Subnet Hosts | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address | Total Hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 124000 | 87.0.0.0 | 255.254.0.0 | 87.0.0.1 | 87.0.0.2 | 87.1.255.254 | 87.1.255.255 | 131070 |
| 90000 | 87.2.0.0 | 255.254.0.0 | 87.2.0.1 | 87.2.0.2 | 87.3.255.254 | 87.3.255.255 | 131070 |
| 80000 | 87.4.0.0 | 255.254.0.0 | 87.4.0.1 | 87.4.0.2 | 87.5.255.254 | 87.5.255.255 | 131070 |
| 20000 | 87.6.0.0 | 255.255.128.0 | 87.6.0.1 | 87.6.0.2 | 87.6.127.254 | 87.6.127.255 | 32766 |

## VLSM Exercise 2: IP = 156.78.0.0 for 20000, 8000, 16000, 500 hosts

Sorted order:
1. 20,000 hosts: Needs 2^15 = 32,768 IPs -> /17 mask (255.255.128.0)
2. 16,000 hosts: Needs 2^14 = 16,384 IPs -> /18 mask (255.255.192.0)
3. 8,000 hosts: Needs 2^13 = 8,192 IPs -> /19 mask (255.255.224.0)
4. 500 hosts: Needs 2^9 = 512 IPs -> /23 mask (255.255.254.0)

| Subnet Hosts | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address | Total Hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 16000 | 156.78.0.0 | 255.255.128.0 (/17) | 156.78.0.1 | 156.78.0.2 | 156.78.127.254 | 156.78.127.255 | 20000 |
| 20000 | 156.78.128.0 | 255.255.192.0 (/18) | 156.78.128.1 | 156.78.128.2 | 156.78.191.254 | 156.78.191.255 | 16000 |
| 8000 | 156.78.192.0 | 255.255.224.0 (/19) | 156.78.192.1 | 156.78.192.2 | 156.78.223.254 | 156.78.223.255 | 8000 |
| 500 | 156.78.224.0 | 255.255.254.0 (/23) | 156.78.224.1 | 156.78.224.2 | 156.78.225.254 | 156.78.225.255 | 500 |

## VLSM Exercise 3: IP = 192.168.1.0 for 50, 12, 30, 19 hosts

Sorted order:
1. 50 hosts: Needs 2^6 = 64 IPs -> /26 mask (255.255.255.192)
2. 30 hosts: Needs 2^5 = 32 IPs -> /27 mask (255.255.255.224)
3. 19 hosts: Needs 2^5 = 32 IPs -> /27 mask (255.255.255.224)
4. 12 hosts: Needs 2^4 = 16 IPs -> /28 mask (255.255.255.240)

| Subnet Hosts | Subnet ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast Address | Total Hosts |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 50 | 192.168.1.0 | 255.255.255.192 (/26) | 192.168.1.1 | 192.168.1.2 | 192.168.1.62 | 192.168.1.63 | 50 |
| 30 | 192.168.1.64 | 255.255.255.224 (/27) | 192.168.1.65 | 192.168.1.66 | 192.168.1.94 | 192.168.1.95 | 30 |
| 19 | 192.168.1.96 | 255.255.255.224 (/27) | 192.168.1.97 | 192.168.1.98 | 192.168.1.126 | 192.168.1.127 | 19 |
| 12 | 192.168.1.128 | 255.255.255.240 (/28) | 192.168.1.129 | 192.168.1.130 | 192.168.1.142 | 192.168.1.143 | 12 |

# 3. TASK 1 — FLSM Static Routing in Packet Tracer

Objective: Configure a Packet Tracer network topology utilizing Class C IP address 196.10.0.0 and default mask 255.255.255.0. We are required to divide it into 5 subnets using FLSM technique. Each subnet is allocated a /27 subnet mask (255.255.255.224), allowing up to 30 usable IP addresses per subnet. Static routing is then configured between the routers to ensure complete connectivity.

Figure 1: Task 1 FLSM Topology Requirements and Instructions

Figure 2: FLSM Exercise Parameters

### Subnet Table for Task 1 (IP: 196.10.0.0 /27 subnets)

| Subnet | Network ID | Subnet Mask | 1st Usable IP | 2nd Usable IP | Last Usable IP | Broadcast IP |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 196.10.0.0 | 255.255.255.224 | 196.10.0.1 | 196.10.0.2 | 196.10.0.30 | 196.10.0.31 |
| 2 | 196.10.0.32 | 255.255.255.224 | 196.10.0.33 | 196.10.0.34 | 196.10.0.62 | 196.10.0.63 |
| 3 | 196.10.0.64 | 255.255.255.224 | 196.10.0.65 | 196.10.0.66 | 196.10.0.94 | 196.10.0.95 |
| 4 | 196.10.0.96 | 255.255.255.224 | 196.10.0.97 | 196.10.0.98 | 196.10.0.126 | 196.10.0.127 |
| 5 | 196.10.0.128 | 255.255.255.224 | 196.10.0.129 | 196.10.0.130 | 196.10.0.158 | 196.10.0.159 |

## Cisco Packet Tracer FLSM Implementation Screenshots

Figure 3: Complete Cisco Packet Tracer Active Simulation Topology for Task 1

Figure 4: IP Address configuration on Router 1 interfaces (Router 4 in Packet Tracer)

Figure 5: IP Address configuration on Router 2 interfaces (Router 5 in Packet Tracer)

Figure 6: PC0 IP configuration window (Subnet 1)

Figure 7: PC2 IP configuration window (Subnet 3)

Figure 8: Command Prompt verifying successful ping test from PC0 to PC2

# 4. TASK 2 — VLSM Static Routing in Packet Tracer

Objective: Configure a Packet Tracer network topology utilizing Class A IP address 10.0.0.0 and default mask 255.0.0.0. We are required to divide this address space into subnets supporting different host counts using VLSM technique:
• Right LAN: 2000 hosts (Needs /21 mask: 255.255.248.0)
• Left LAN: 1000 hosts (Needs /22 mask: 255.255.252.0)
• Middle LAN: 400 hosts (Needs /23 mask: 255.255.254.0)
• WAN Link: 2 hosts (Needs /30 mask: 255.255.255.252)
Static routing is established across routers to connect these multi-host networks.

Figure 9: Task 2 VLSM Topology Requirements and Host Requirements

Figure 10: VLSM Design Rules and Guidelines

## VLSM Subnet Allocation Details

### LAN 1: Right LAN (2000 Hosts Needed)

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.0.0 |
| Subnet Mask | 255.255.248.0 (/21) |
| First Usable IP | 10.0.0.1 |
| Last Usable IP | 10.0.7.254 |
| Broadcast IP | 10.0.7.255 |
| Total Hosts Supported | 2046 |

### LAN 2: Left LAN (1000 Hosts Needed)

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.8.0 |
| Subnet Mask | 255.255.252.0 (/22) |
| First Usable IP | 10.0.8.1 |
| Last Usable IP | 10.0.11.254 |
| Broadcast IP | 10.0.11.255 |
| Total Hosts Supported | 1022 |

### LAN 3: Middle LAN (400 Hosts Needed)

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.12.0 |
| Subnet Mask | 255.255.254.0 (/23) |
| First Usable IP | 10.0.12.1 |
| Last Usable IP | 10.0.13.254 |
| Broadcast IP | 10.0.13.255 |
| Total Hosts Supported | 510 |

### WAN Link: Serial Link (2 Hosts Needed)

| Field | Value |
| --- | --- |
| Subnet ID | 10.0.14.0 |
| Subnet Mask | 255.255.255.252 (/30) |
| First Usable IP | 10.0.14.1 |
| Last Usable IP | 10.0.14.2 |
| Broadcast IP | 10.0.14.3 |
| Total Hosts Supported | 2 |

## VLSM Subnet Summary Table

| Network Block | Purpose / Location | Assigned Subnet Range |
| --- | --- | --- |
| 10.0.0.0/21 | Right LAN (2000 hosts) | Router interface → Switch Local |
| 10.0.8.0/22 | Left LAN (1000 hosts) | Router interface → Switch Local |
| 10.0.12.0/23 | Middle LAN (400 hosts) | Switch shared segment |
| 10.0.14.0/30 | WAN serial link | Router ↔ Router interconnect |

## Cisco Packet Tracer VLSM Implementation Screenshots

Figure 11: Task 2 VLSM Topology Setup in Cisco Packet Tracer

Figure 12: IP Address configuration on Router 1 interfaces (Router 4 in Packet Tracer)

Figure 13: IP Address configuration on Router 2 interfaces (Router 5 in Packet Tracer)

Figure 14: PC0 IP configuration window (Left LAN)

Figure 15: PC4 IP configuration window (Right LAN)

Figure 16: Verification ping from PC0 to PC4 showing network convergence

Figure 17: Verification ping from PC4 to PC0 showing two-way reachability